Batch:   TY A2            Roll No.:  16010122041

Experiment / assignment / tutorial No.___2___

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Experiment No. 2

Title: Study of basic network administration commands and network configuration.

AIM: Study networking commands –ping, traceroute, nslookup, arp, rarp, netstat, telnet.

_____________________________________________________________________

Expected Outcome of Experiment:

- Understand the fundamentals of network administration.

_____________________________________________________________________

Books/ Journals/ Websites referred:

- Linux Lab - Open source Technology : Ambavade –Dreamtech

- http://manpages.ubuntu.com/manpages/trusty/man8/rarp.8.html

- http://computernetworkingnotes.com/comptia-n-plus-study-guide/network-tool-command.html

____________________________________________________________________

Pre Lab/ Prior Concepts: Computer Network

__________________________________________________________________

New Concepts to be learned:   Command line operation to handle networks.

__________________________________________________________________

Computers are connected in a network to exchange information or resources each other. Two or more computer connected through network media called computer network. There are number of network devices or media are involved to form computer network. Computer loaded with Windows and Linux Operating System can also be a part of network whether it is small or large network by its multitasking and multiuser natures. Maintaining of system and network up and running is a task of System / Network Administrator’s job.

Frequently used network configuration and troubleshoot commands in Linux/Windows are as follows:

1. IFCONFIG/ IPCONFIG

ifconfig (interface configurator) command is use to initialize an interface, assign IP Address to interface and enable or disable interface on demand. With this command you can view IP Address and Hardware / MAC address assign to interface and also MTU (Maximum transmission unit) size.

ifconfig with interface (eth0) command only shows specific interface details like IP Address, MAC Address etc. with -a options will display all available interface details if it is disable also.

Syntax:                               # ifconfig  eth0

To enable or disable specific Interface, we use example command as follows.

Enable eth0:                     # ifup eth0

Disable eth0:                    # ifdown eth0

To Setting MTU Size:

By default, MTU size is 1500. We can set required MTU size with below command.

Replace XXXX with size.

Syntax:                                # ifconfig eth0 mtu XXXX

Set Interface in Promiscuous mode.

Network interface only received packets belongs to that particular NIC. If you put interface in promiscuous mode, it will receive all the packets. This is very useful to capture packets and analyse later. For this you may require superuser access.

Syntax:                               # ifconfig eth0 - promisc

2. PING

PING (Packet INternet Groper) command is the best way to test connectivity between two nodes. Whether it is Local Area Network (LAN) or Wide Area Network (WAN). Ping use ICMP (Internet Control Message Protocol) to communicate to other devices.

It verifies IP-level connectivity to another TCP/IP computer by sending Internet Control Message Protocol (ICMP) Echo Request messages. The receipt of corresponding Echo Reply messages are displayed, along with round-trip times. Ping is the primary TCP/IP command used to troubleshoot connectivity, reachability, and name resolution.

ping [-c count] [-i wait] [-l preload][-s packetsize] host

-c count

Stop after sending (and receiving) count ECHO_RESPONSE packets.

-i wait

Wait wait seconds between sending each packet.  The default is to

wait for one second between each packet.  This option is

incompatible with the -f option.

-l preload

If preload is specified, ping sends that many packets as fast as

possible before falling into its normal mode of behavior.

-s packetsize

Specifies the number of data bytes to be sent.  The default is

56, which translates into 64 ICMP data bytes when combined with

the 8 bytes of ICMP header data.

PING Command Example:

# ping 4.2.2.2

# ping -c 5 www.tecmint.com

3.  TRACEROUTE/ TRACERT

traceroute is a network troubleshooting utility which shows number of hops taken to reach destination also determine packets traveling path. Below we are tracing route to global DNS server IP Address and able to reach destination also shows path of that packet is traveling.

Syntax:

tracert [-d] [-h MaximumHops] [-j HostList] [-w Timeout] [TargetName]

Parameters

-d : Prevents tracert from attempting to resolve the IP addresses of intermediate routers to their names. This can speed up the display of tracert results.

-h:  MaximumHops Specifies the maximum number of hops in the path to search for the target (destination). The default is 30 hops.

-j: HostList Specifies that Echo Request messages use the Loose Source Route option in the IP header with the set of intermediate destinations specified in HostList.The HostList is a series of IP addresses (in dotted decimal notation) separated by spaces.

-w : Timeout Specifies the amount of time in milliseconds to wait for the ICMP Time Exceeded or Echo Reply message corresponding to a given Echo Request message to be received. If not received within the time-out, an asterisk (*) is displayed. The default time-out is 4000 (4 seconds).

4. NETSTAT command

Displays active TCP connections, ports on which the computer is listening, Ethernet statistics, the IP routing table, IPv4 statistics (for the IP, ICMP, TCP, and UDP protocols), and IPv6 statistics (for the IPv6, ICMPv6, TCP over IPv6, and UDP over IPv6 protocols).

Netstat provides statistics for the following:

Proto - The name of the protocol (TCP or UDP).

Local Address - The IP address of the local computer and the port number being used. The name of the local computer that corresponds to the IP address and the name of the port is shown unless the -n parameter is specified. If the port is not yet established, the port number is shown as an asterisk (*).

Foreign Address - The IP address and port number of the remote computer to which the socket is connected. The names that correspond to the IP address and the port are shown unless the -n parameter is specified. If the port is not yet established, the port number is shown as an asterisk (*).

(state) Indicates the state of a TCP connection. The possible states are as follows:

CLOSE_WAIT

CLOSED

ESTABLISHED

FIN_WAIT_1

FIN_WAIT_2

LAST_ACK

LISTEN

SYN_RECEIVED

SYN_SEND

TIMED_WAIT

Syntax

netstat [-a] [-e] [-n] [-o] [-p Protocol] [-r] [-s] [Interval]

Parameters

Used without parameters, netstat displays active TCP connections.

-a Displays all active TCP connections and the TCP and UDP ports on which the computer is listening.

-e Displays Ethernet statistics, such as the number of bytes and packets sent and received. This parameter can be combined with -s.

-n Displays active TCP connections, however, addresses and port numbers are expressed numerically, and no attempt is made to determine names.

-o Displays active TCP connections and includes the process ID (PID) for each connection.

-p Shows connections for the protocol specified by Protocol.

-s Displays statistics by protocol. By default, statistics are shown for the TCP, UDP, ICMP, and IP protocols. If the IPv6 protocol for Windows XP is installed, statistics are shown for the TCP over IPv6, UDP over IPv6, ICMPv6, and IPv6 protocols. The -p parameter can be used to specify a set of protocols.

-r Displays the contents of the IP routing table.

Netstat (Network Statistic) command display connection info, routing table information etc. To displays routing table information use option as -r.

# netstat –r

5.  DIG

Dig (domain information groper) query DNS related information like A Record, CNAME, MX Record etc. This command mainly uses to troubleshoot DNS related query.

# dig   www. Ipadress.com

6. NSLOOKUP

The name "nslookup" means "name server lookup". nslookup is a network administration command-line tool available for many computer operating systems for querying the Domain Name System (DNS) to obtain domain name or IP address mapping or for any other specific DNS record. It displays information from Domain Name System (DNS) name servers.

nslookup command also use to find out DNS related query.

Example:

C:\Documents and Settings\sysadm>nslookup itu.dk

Server:  ns3.inet.tele.dk

Address:  193.162.153.164

Non-authoritative answer:

Name:    itu.dk

Address:  130.226.133.2

# nslookup   www. Googel.com

7. ROUTE

Route command also shows and manipulate ip routing table. To see default routing table in Linux, type the following command.

# route

8. ARP

When we need an Ethernet (MAC) address we can use arp(address resolution protocol).

In other words it shows the physical address of an host.

Syntax

arp [-a [InetAddr] [-N IfaceAddr]] [-g [InetAddr] [-N IfaceAddr]] [-d InetAddr [IfaceAddr]] [-s InetAddr EtherAddr [IfaceAddr]]

Parameters

Used without parameters, ping displays help

-a [InetAddr] [-N IfaceAddr] Displays current ARP cache tables for all interfaces.

-g [InetAddr] [-N IfaceAddr] Identical to -a.

-d InetAddr [IfaceAddr] Deletes an entry with a specific IP address, where InetAddr is the IP address.

-s InetAddr EtherAddr [IfaceAddr] Adds a static entry to the ARP cache that resolves the IP address InetAddr to the physical address EtherAddr.

To add a static ARP cache entry to the table for a specific interface, use the IfaceAddr parameter where IfaceAddr is an IP address assigned to the interface

ARP (Address Resolution Protocol) is useful to view / add the contents of the kernel’s ARP tables. To see default table use the command as.

# arp -e

Address                  HWtype  HWaddress           Flags Mask            Iface

192.168.50.1             ether   00:50:56:c0:00:08   C                     eth0

9 . ETHTOOL

ethtool is a replacement of mii-tool. It is to view, setting speed and duplex of your Network Interface Card (NIC). You can set duplex permanently in /etc/sysconfig/network-scripts/ifcfg-eth0 with ETHTOOL_OPTS variable.

Syntax: # ethtool eth0

10. TELNET

The telnet command is used to communicate with another host using the TELNET protocol. If telnet is invoked without the host argument, it enters command mode, indicated by its prompt (telnet> ) In this mode, it accepts and executes the commands listed below. If it is invoked with arguments, it performs an open command with those arguments.

To login to a remote machine, use this syntax:

% telnet <hostname>

The options are as follows:

-8 Specifies an 8-bit data path. This causes an attempt to negotiate the TELNET BINARY option on both input and output.

-E Stops any character from being recognized as an escape character.

-K Specifies no automatic login to the remote system.

11. HOSTNAME

hostname is to identify in a network. Execute hostname command to see the hostname of your box. You can set hostname permanently in /etc/sysconfig/network. Need to reboot box once set a proper hostname.

# hostname

12. SYSTEMINFO

Display information about a system.

IMPLEMENTATION:

Show the use of different network commands:

- ipconfig

- ping

- traceroute

- netstat

- nslookup

- route

- arp

- hostname

- systeminfo

CONCLUSION:

We have learned about networking commands such as ping, traceroute, nslookup, arp, rarp, netstat, and telnet, which provide essential tools for effective network troubleshooting and management.